Unofficial Advanced Gravis Ultrasound Drivers #2 ("Manley Drivers")

Copyright 1994 Robert J. Manley and Multi-Player Computer Adventures

Version 0.6c - SHAREWARE - please read the REGISTRATION section for details

FEATURES

- basic GUS rev 3.7 and MAX/daughtercard mixer support

- 8 or 16 bit wavefile playback

- mono or stereo

- hardware MIDI playback (to an external synth or synth module)

- volume and balance control

- multi-session (many registered streams - one active)

- Ultrasound specific OS/2 VDD giving DOS sessions:

- reduced interrupt latency

- hardware virtualization

- MIDI port emulation of Roland MPU-401 in UART mode

(allowing WinOS2 to load and use those drivers - hooray!)

- GM emulation for DOS and Windows sessions under OS/2 with

the goody pack which is only sent to registered users

- absolute lowest system overhead possible

- rigorous testing

Here is what it has been tested and known to work with:

- OS/2 2.1 with MMOS2 CSD, OS/2 2.11 (with its MMOS2 CSD), Warp!

- system sounds

- digital audio applet

- MIDI applet

- compact disc applet and CD explorer (bug in version 2.3 hangs

PM message queue when using digital transfer)

- bigben2

- dmplayer 1.01

- video player / Ultimotion

- wave doctor

- multimedia mahjongg

- GUSmixer (even while the device is in use!)

- play.cmd (thanks to TFS for reminding me about this one)

- WinOS2 apps

- media player

- Cakewalk Pro

- Midisoft Recording studio

INSTALLATION

*** STEP #1 ***

This driver requires one of the following to be installed:

- OS/2 Warp (with MMOS2 installed) Beta I, Beta II, or GA

- OS/2 2.11 MMOS2 from either

- OS/2 2.11 refresh package

- OS/2 2.1 + Feb '94 Service Pack (== OS/2 2.11)

- OS/2 2.1 MMOS2 + November '93 MMOS2 CSD

(ftp-os2 in pub/os2/ibm/os2fixes as MMPMCS.DSK)

If neither of these versions of MMOS2 can be found, the installation will

fail and nothing will be added to your CONFIG.SYS.

*** STEP #2 ***

Use an OS/2 based unzip program (e.g. Info-Zip's) to unpack ULTRA05E.ZIP to a

directory on your hard disk or floppy.  From that directory, run "MINSTALL",

accept it as the source, and then proceed with the installation.  The following

lines will be appended to your CONFIG.SYS:

DEVICE=<drive>:\MMOS2\ULTRADD.SYS /B:b /C:c /D:d /U:u /M:m /V

DEVICE=<drive>:\MMOS2\ULTRAVDD.SYS

where (with defaults):

b - base I/O address of Ultrasound (220)

c - base address of 16-bit CODEC (32C)

d - DMA channel for playback (7)

u - GF1 IRQ (12)

m - MIDI IRQ (5)

If you have a 16-bit daughtercard, you must manually change the address (in

hex) following the /C: switch to the base address for that board.  The valid

addresses for the daughtercard are: 530, 604, E80, and F40.  You may install

this software over a previous version without first uninstalling.  Also note

that the driver can be UNINSTALLED by selecting the number of Ultrasounds to

install as zero.

*** IMPORTANT STEP #3 (NEW) ***

If you have previously installed an earlier version of this driver since

installing your current system, you must read this step carefully and follow

all instructions.  There is a limitation in the MMOS2 installation program

which forces the user to manually edit the CONFIG.SYS.  After installing this

software, search for a line containing the token "AUDIOVDD" such as the

following:

DEVICE=<drive>:\MMOS2\AUDIOVDD.SYS ULTRA1$

If this line is present as shown here, comment it out with a REM statement at

the beginning.  If you have another OS/2 supported soundcard installed and

this line contains other tokens (e.g. "PAS161$"), try these two things in

order of preference:

1. delete just the "ULTRA1$" token from this line and install

2. after installing this software and experiencing problems with

GUS sound in DOS boxes, put back the "ULTRA1$" token and REM

out the ULTRAVDD.SYS line.  Sorry, no emulations for you.

If I had more than one soundcard installed, I could check this but I don't :(.

HARDWARE EMULATIONS

In addition to virtualizing the Ultrasound I/O ports to DOS sessions, the

supplied OS/2 virtual device driver (VDD) ULTRAVDD.SYS also emulates the

following devices when selected in the AUDIO_ADAPTER_EMULATION DOS setting:

- Roland MPU-401 MIDI interface in UART mode (e.g. Windows).

Select I/O address of 0x330 and IRQ of 2/9 (not used yet).

MIDI is re-routed through the GUS MIDI port to an external synth

(not played natively as in MegaEm).

**** NOTE: native DOS GM emulation is now done! (like MegaEm for OS/2)

**** Register this software and get the files required to enable this

**** functionality by e-mail or if neccessary, ground mail

Future directions are to add Sound Blaster and enhance Roland emulation.

Please note that neither SBOS or MegaEm should be run under an OS/2 DOS

box.  These programs require features of the 386 that cannot be granted

to a VDM under a protect mode OS without comprimising system integrity.

UNDER NO CIRCUMSTANCES SHOULD SBOS or MEGAEM BE RUN!

WinOS2 CONFIGURATION

It is recommended that all WinOS2 users keep a separate copy of their

SYSTEM.INI and CONTROL.INI files for real Windows and WinOS2.  Doing this

will allow the Gravis supplied Windows drivers to work while under real

Windows and these drivers to provide MPU-401 emulation under WinOS2.  A

good system to use is to copy each of these files to another with the same

filename root but with the extension ".DOS" after a successful run of real

Windows.  Do the same with the extension ".OS2" after running this setup

without problems.  Before starting either system, copy the OS specific

versions over their proper files.  PLEASE BE CAREFUL with this and take

the time to understand what you are actually doing!

The first thing to do is get rid of all references to the Gravis drivers

in your Window's SYSTEM and CONTROL ini files (of course, this is only

after you've already made backup copies of these files for real Windows

as described above).

These are the lines in your real Windows SYSTEM.INI to comment out with a

';' character at the beginning to disable Gravis' drivers and avoid that

stupid error message listed in the QUESTIONS section below:

device=grvsultr.386

midi=ultrasnd.drv

...

midi1=ultmport.drv

wave=ultrasnd.drv

AUX=ultrasnd.drv

mixer=ultrasnd.drv

These are the lines in your CONTROL.INI file to do the same with:

ultmport.drv=UltraSound MIDI Port

ultrasnd.drv=Ultra Wave and MIDI Synth

...

midi=ultrasnd.drv

midi1=ultmport.drv

wave=ultrasnd.drv

AUX=ultrasnd.drv

The next thing to change are the parameters Windows uses for the system

timer which drives the MIDI playback.  Search for the following block in

your SYSTEM.INI:

[timer.drv]

TIMERMax386Res=10

TIMERMax286Res=10

Change both values from 10 to 1.

Next fire up WinOS2 and install the Roland MPU-401 drivers through the

"Drivers" setup inside Control Panel.  Now go into the MIDI Mapper and

create a setup called "UltraVDD".  Select the Roland MPU-401 device as

the destination port for all MIDI channels and then click on "OK" to

complete the setup.

If you are running full OS/2 (with IBM's WinOS2), you may wish to copy

the TIMER.DRV from your real Windows SYSTEM directory to the equivalent

WinOS2 directory after renaming the original.  This driver seems to give

more accurate MIDI timing for Windows based MIDI sequencers.

KNOWN BUGS AND LIMITATIONS

- moving Ultimotion window can generate noise or loss of sound

(this seems to be fixed in Warp)

- the Digital Audio, MIDI, and Digital Video icons in the multimedia folder

are replaced with incorrect versions (these can be easily changed back

manually)

- the installation program is lacking in functionality (see STEP #3 in the

INSTALLATION section)

- MPU-401 interrupt driven recording isn't working yet (booo, hiss)

- no text in Description field for AUDIO_ADAPTER_EMULATION property

(this is a VDM limitation of OS/2 on Intel platform)

MOST COMMONLY ASKED QUESTIONS

Q. Why don't I hear anything when using the OS/2 MIDI applet?

A. If you don't have a MIDI synth connected, you won't.

A. Why don't I hear anything when using WinOS2 MIDI applications?

A. If you are not a registered user and don't have a MIDI synth connected,

you won't.

A. If you do have a MIDI synth, check the WinOS2 CONFIGURATION section above.

A. If you are a registered user, please read the WinOS2 CONFIGURATION section

in this document and the INSTALLATION instructions included with your GM

"goody pack".

Q. Why is there a delay before hearing a system sound?

A. OS/2 must swap out blocks of memory to give the MMOS2 subsystem the

storage that it needs for holding the sound sample.  When enough memory

is already free (such as immediately after a system sound), the delay

should be almost unnoticeable.

Q. Why do I get the error message "The Ultrasound driver requires enhanced

mode to run" when trying to start a full-screen WinOS2 session?

A. Gravis' drivers include a Windows Virtual Device Driver (VxD != OS/2 VDD)

which cannot be loaded by OS/2 for system integrity reasons.  When the

load fails, WinOS2 (and Windows) interpret this error as the system not

running in 386 enhanced mode since that is the mode required for loading

and using VxDs.

Q. Why do my seamless WinOS2 sessions just sit there and never start?

A. See Q and A above.  When the VxD load fails, WinOS2 can't continue nor

can it properly exit.  Use a session killer like Watchcat to kill it.

Q. Why doesn't SBOS or MegaEm work?

A. Please read the HARDWARE EMULATIONS section.

LEGALESE

This software is provided AS IS.  No warranty of any kind, expressed or

implied, is granted including any warranties of quality or fitness for

a particular purpose.  Under no circumstances will either Robert J. Manley

or Multi-Player Computer Adventures be held responsible for any damage

that results directly or indirectly from the use of this product.

REGISTRATION

Please fill in the following information and send it along with a cheque

or money order for $25.00 US or CAN (we need the break) to:

Multi-Player Computer Adventures

14845-6 Yonge Street, Box 314

Aurora, Ontario, Canada

L4G 6H8

Name: __________________________________________________

Company: __________________________________________________

Address: __________________________________________________

: __________________________________________________

: __________________________________________________

e-mail: __________________________________________________

phone: __________________________________________________

Price includes 7% GST for Canadians and 8% PST for residents of Ontario.

Some future versions of this product may only be distributed commercially

but will be sent free to all registered users.

BUG REPORTS

*** NEW e-mail address ***

Please send all bug reports, gratuitous worship, whatever, to me through

e-mail at: rjm@dciem.dciem.dnd.ca.  I enjoy hearing from all users.

Problem symptom: _____________________________________________

Fix or workaround: _____________________________________________

OS/2 version: ______

GUS hardware revision: ___

Daughtercard? (y/n): _

External MIDI? (y/n): _

GUS memory: ____ K

System memory: __ M

WHAT'S NEXT

- Sound Blaster emulation to DOS sessions

- multi-track mod support at the driver level (need help here)

- MIDI synth

- Digital Audio recording

- getting Advanced Gravis to licence this software from me

FINALLY

I cannot emphasize enough my thanks to Brent Davis and especially Linden

deCarmo who have provided invaluable information when I needed it most.

Enjoy.  Robert.